ESP32-S3 + A7672S (4G GNSS) — Full Setup Guide

1) Hardware overview

You’re using an ESP32-S3 board with an integrated SIMCom A7672S.

Ports on the board

- ESP32PRG → native USB (CDC/JTAG)

- HOST → USB-UART bridge (reliable COM for upload/serial)

- 4GUSB → direct modem USB (AT/NMEA only)

For Arduino work:

- Use HOST (COMx) for upload + Serial Monitor

2) Required tools

Arduino IDE setup

- Add ESP32 board URL:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Install:

esp32 by Espressif Systems

- Select board:

ESP32S3 Dev Module

3) Important Arduino settings

Set these exactly:

USB Mode → USB-Serial/JTAG (for easy upload)

USB CDC On Boot → ENABLED

Upload Speed → 921600

If upload fails → use BOOT button method

4) SIM & Antenna setup

SIM

- Insert SIM (data enabled)

- Disable SIM PIN

Antennas

- 📶 LTE antenna → tower symbol port

- 🛰️ GPS antenna → satellite port

5) Basic AT communication test

Upload simple passthrough code:

void setup() {

Serial.begin(115200);

Serial2.begin(115200, SERIAL_8N1, 18, 17);

}

void loop() {

if (Serial.available()) Serial2.write(Serial.read());

if (Serial2.available()) Serial.write(Serial2.read());

}

Then type:

AT

AT+CPIN?

AT+CSQ

6) Network setup (4G)

AT+CGDCONT=1,"IP","airtelgprs.com"

AT+CGATT=1

AT+CGACT=1,1

Check:

AT+CGPADDR=1

7) GPS setup

AT+CGNSSPWR=1

Get location:

AT+CGPSINFO

8) Firebase setup

- Use Realtime Database

- Rules:

{

"rules": {

".read": true,

".write": true

}

}

9) HTTPS POST (WORKING FLOW ✅)

You confirmed this works:

// Create JSON

String json = "{\"lat\":" + String(latitude,6) +

",\"lon\":" + String(longitude,6) + "}";

String url = "https://bus-tracking-eae81-default-rtdb.asia-southeast1.firebasedatabase.app/gps.json";

// 1. Clean previous session

sendAT("AT+HTTPTERM");

delay(500);

// 2. Init HTTP

sendAT("AT+HTTPINIT");

// 3. Set parameters

sendAT("AT+HTTPPARA=\"URL\",\"" + url + "\"");

sendAT("AT+HTTPPARA=\"CONTENT\",\"application/json\"");

// 4. Enable SSL (CRITICAL for Firebase)

sendAT("AT+HTTPPARA=\"SSLCFG\",0");

// 5. Send data

Serial2.println("AT+HTTPDATA=" + String(json.length()) + ",5000");

delay(1000);   // wait for DOWNLOAD

Serial2.print(json);

delay(500);

// 6. POST

String response = sendAT("AT+HTTPACTION=1", 6000);

// 7. Cleanup

sendAT("AT+HTTPREAD");

sendAT("AT+HTTPTERM");

10) Your success output (confirmed)

+HTTPACTION: 1,200,31

✅ Firebase UPDATE SUCCESS

This means:

- HTTPS working ✅

- SSL configured ✅

- Data stored in Firebase ✅

11) Common errors & fixes

❌ HTTPINIT ERROR

Fix:

AT+HTTPTERM

AT+HTTPINIT

❌ CID ERROR

Ignore — your firmware doesn’t require CID

❌ 706 error

Means network not linked → fix APN / CGACT

❌ No GPS data

Go outside, wait 30–60 sec

❌ No COM port

Use HOST port
 Enable USB CDC
 Check cable

12) Final system architecture

GPS (GNSS)

↓

A7672S Modem

↓

ESP32 (UART)

↓

HTTPS POST

↓

Firebase Realtime DB

↓

Mobile App (next step 🚍)

🚍 What you have built

- GNSS tracking ✅

- 4G connectivity ✅

- HTTPS communication ✅

- Cloud integration (Firebase) ✅

This is already a complete IoT tracking backend